ROOF REPLACEMENT AT THE CHEROKEE NATION MARSHALL BUILDING

October 7th, 2026

Clarification to the roof specification #2

This addendum is hereby a part of those documents and contracts, the same as if bound therein and it shall supersede anything contained in the original bid manual with which it might conflict.

- At Marshall Building:

-For alternate system, flashing backer sheet and flashing membrane shall both be set in flashing adhesive prior to heat welding side laps and cant/membrane transition of flashing backer sheet and flashing membrane.

-At curbs to be covered with metal cap, provide slope in metal cap to ensure positive drainage toward scuppers.

-Include as an add-alternate for both base bid and alternate bid: Contractor shall provide and install new ¾” condensation lines and new 2” main lines that condensation lines will tie into. No more than five (5) condensation lines are to tie into a single 2” main line. New main lines are to run the full length to the nearest scupper in which the downspout drains underground. Main lines shall not run to a scupper where the connecting downspout drains onto the pavement. New condensation and main line piping shall be Type L Copper. Traps shall be provided by the owner and installed by the owner.

-Owner shall install three (3) roof hydrants once existing roof is removed down to the steel decking. Owner will be responsible for cutting three (3) holes into steel decking and installing roof hydrants. Contractor shall install pitch pans around each roof hydrant. Set hydrant in non-shrink grout inside pitch pan. Fill top with pourable sealer. Cap pitch pan with new hood. Install band on top of hood and seal band with sealant as specified.

-At curb and wall transitions, omit new horizontal wood blocking referenced in detail drawings. See revised detail drawings below for reference.