Statement of Work

| Date: | 9/24/2026 |
| --- | --- |
| Client: | Roland Travel Plaza |
| Job Name: | ROL – Truck Diesel Dispenser Replacement |
| Requested by: | Ron Phillips |
| Addendum: |  |

The Cherokee Nation Entertainment is seeking proposals for the purchase, delivery, and installation of new Diesel Dispensers at the Roland Travel Plaza. Qualified vendors (“bidders”) are requested to provide bids to supply the equipment and related services as outlined in this document.

Bidders shall be able to guarantee service and response time by providing onsite technical services, customer support, and telephone support.

- Summary

This is for procurement and installation of new diesel dispensers at the Roland Travel Plaza.

This work consists of but is not limited to installing all equipment per specification, electrical disconnect and removal of existing equipment, prep piping, installation of new dispensers, electrical connection of new dispensers, start up, programming and purging of each dispenser.

- Project Scope

| 1 | Wayne HS11/1220D Dispensers - Valance Accessories List: Valance Kit: WU010194-0591Wireless Wayne Connect Kit: DFS Wireless Connect subscriber kit: WU019855-0001Site is activated at start-up for outdoor EMV transactions: Yes (Dispensers will be shipped with EMV Software)User/Payment Interface: Anthem or AX12 Platforms Additional Payment Options: Wayne TAP / NFC Reader Other User Interface Options: Satellite-in-Use Indicator Connectivity Options: DFS Wireless Connect subscriber unit Secure EMV Hybrid Card Reader Selection: XAC P95W card reader Additional Payment Options: Touchscreen Alphanumeric Keypad UX Display Type: AX12Media Options: DX Promote Media Site Media Status - DX Promote: Site does not have an existing DX Promote Media Subscription for this UX Display type Identify Site Brand Image: Wayne Standard Main Bezel Color: A017 Silver U-Frame Bezel Color: A017 Silver Column Color: A017 Silver Hydraulic Door Option: A017 Silver Lower Column Skin: A017 Silver Right Nozzle Boot Adapter: A017 Silver Blank Nozzle Boot Adapter: A017 Silver High Speed Master 1 Product PTS Decal: Diesel iX Software: EMV Anthem SPM 2.5 keypad: W2894444-202-700 WAYNE GENERIC Site Brand: Wayne Generic Valance: Ovation2 Valance Standard 54.2"Valance Option: R098 Red HS Filtration: Dual 40-GPM Filters per Hose Extended Warranty: 2 Years Additional Media Only Warranty - AX12 with DX Promote Auto |
| --- | --- |
| 4 | Wayne HS12/1220C Dispensers - Valance Accessories List: Valance Kit J021 Black: WU010194- J021Wireless Wayne Connect Kit: DFS Wireless Connect subscriber kit: WU019855-0001Site is activated at start-up for outdoor EMV transactions: Yes (Dispensers will be shipped with EMV Software)User/Payment Interface: Anthem or AX12 Platforms Additional Payment Options: Wayne TAP / NFC Reader Other User Interface Options: Satellite-in-Use Indicator Connectivity Options: DFS Wireless Connect subscriber unit Secure EMV Hybrid Card Reader Selection: XAC P95W card reader UX Display Type: AX12Additional Payment Options: Touchscreen Alphanumeric Keypad Media Options: DX Promote Media Site Media Status - DX Promote: Site does not have an existing DX Promote Media Subscription for this UX Display type Identify Site Brand Image: Wayne Standard Main Bezel Color: A017 Silver U-Frame Bezel Color: A017 Silver Column Color: A017 Silver Lower Column Skin: A017 Silver Right Nozzle Boot Adapter: A017 Silver Blank Nozzle Boot Adapter: A017 Silver Hydraulic Door Option: R098 Red High Speed Master 1 Product PTS Decal: Diesel Satellite Side B- Right Nozzle Product ID: Diesel #2iX Software: EMV Anthems 2.5 keypad: W2894444-202-700 WAYNE GENERIC Site Brand: Wayne Generic Valance: Ovation2 Valance Standard 54.2"Valance Option: J021 Black HS Filtration: Dual 40-GPM Filters per Hose Extended Warranty: 2 Years Additional Media Only Warranty - AX12 with DX Promote Auto |  |
| 1 | Wayne HS11/1120S Dispenser Satellite Only - Valance Accessories List: Valance Kit J021 Black: WU010194-J021User/Payment Interface: No Payment Terminal Identify Site Brand Image: Wayne Standard Main Bezel Color: A017 Silver U-Frame Bezel Color: A017 Silver Column Color: A017 Silver Lower Column Skin: A017 Silver Right Nozzle Boot Adapter: A017 Silver Blank Nozzle Boot Adapter: A017 Silver Hydraulic Door Option: J021 Black High Speed Satellite 1 Product Nozzle Decal: Diesel #2Site Brand: Wayne Generic Valance: Ovation2 Valance Standard 54.2"Valance Option: J021 Black |  |
| 1 | Wayne Site Accessory Kit - Remote Control: Remote Control: WM002290Solutions Gateways: DFS Edge IoT Gateway: WU027165-0001Connectivity: DFS Wireless Connect Access Point: WU019854- 0001Solutions Set Up Fees: SaaS On-boarding Fee: SAAS-SETUP FEEDX Promote Subscriptions: DX Promote Auto for AX12 - 4 Yr subscription: W2894381-012 |  |
| 8 | CIM 800-10 1" Filter |  |
| 11 | Hose Kit |  |
| 1 | Misc. Dispenser installation Materials |  |

Components

- Contractor Provided

- Contractor will provide all miscellaneous components to complete the install.

- Electrical disconnect of existing dispensers

- Load up new dispensers and remove and dispose of existing dispensers, pursuant to applicable federal, state and local laws and regulations.

- Prep piping and installing new dispensers

- Electrical connection of new dispensers

- Start up, programming, and purging of each dispenser

Summary of Requirements

Conditions:

- Work schedule will be coordinated with CNE.

Key Requirements

- Contractors must keep all work areas always clean of debris.

- Contractor has all necessary tools to accomplish tasks associated with the project.

- No substitutions are allowed unless owner permission is provided in writing.

- Any change orders must be agreed upon and signed off on by the CNE Engineer & CNE Manager.

- Vendor must provide insurance requirements including Pollution Liability insurance.