Milam Home Plant Install Scope of Work

Spray existing beds to kill all vegetation

Remove all existing plant material in preparation for planting

Till beds and add Growers Mix, fertilizer & new soil as needed

Install and secure weed barrier cloth in all flower beds

Plant all plants and trees as shown on landscape drawings.

Install new trees 2” above grade and use 2-3 t posts and nylon straps as needed

Mulch all beds with non-float cypress mulch

Provide all labor and equipment to complete the bed clean out and planting of all plants and trees.

Install new sod on all lawn areas that get disturbed

Irrigation is existing, repair all irrigation if damaged during the plant installation

During the work progression, the premises are to be kept neat and orderly at all times

Protect existing fences, utilities, sidewalks, and other facilities from damage caused by landscape operations